# Hardware Connection Details

This document describes the complete hardware connections for the Sports Equipment Monitoring System using Arduino Uno, MPU6050 gyroscope, ADXL345 accelerometer, HC-05 Bluetooth module, and a 16x2 LCD display.

## 1. Arduino Uno

Arduino Uno acts as the central microcontroller. It interfaces with sensors and LCD using the I2C protocol and communicates wirelessly via Bluetooth.

## 2. MPU6050 (Gyroscope)

| MPU6050 Pin | Arduino Uno Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

The MPU6050 uses the I2C protocol. SDA and SCL lines are shared with other I2C devices.

## 3. ADXL345 Accelerometer (I2C Mode)

| ADXL345 Pin | Arduino Uno Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |
| CS | 3.3V (I2C Mode) |
| SDO | GND (I2C Address 0x53) |

The ADXL345 operates at 3.3V and communicates using I2C protocol.

## 4. HC-05 Bluetooth Module

| HC-05 Pin | Arduino Uno Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TXD | RX (D0) / SoftwareSerial RX |
| RXD | TX (D1) via voltage divider |

The HC-05 module enables wireless communication. A voltage divider is recommended on RXD pin.

## 5. 16x2 LCD Display (I2C Module)

| LCD Pin | Arduino Uno Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

The 16x2 LCD with I2C module displays sensor readings and system status. It shares the I2C bus with MPU6050 and ADXL345.